CROSSOVER WORKSHEET

|  | ## Site |  |  |  |  | ## Date |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Film Type |  |  |  |  | ## Technologist |  |  |  |
| ## New Emulsion # |  |  |  |  | ## Old Emulsion # |  |  |  |  |
| Film # | Low Density (LD) Step # | Mid Density (MD) Step # | High Density (HD) Step # | B+F | # Film # | Low Density (LD) Step # | Mid Density (MD) Step # | High Density (HD) Step # | B+F |
| 1 |  |  |  |  | 1 |  |  |  |  |
| 2 |  |  |  |  | 2 |  |  |  |  |
| 3 |  |  |  |  | 3 |  |  |  |  |
| 4 |  |  |  |  | 4 |  |  |  |  |
| 5 |  |  |  |  | 5 |  |  |  |  |
| Average |  |  |  |  | Average |  |  |  |  |
| Average Density Difference: DD = HD – LD = |  |  |  |  | Average Density Difference: DD = HD – LD = |  |  |  |  |
|  |  | MD difference between new and old film (New MD – Old MD) |  |  |  |  |  |  |  |
|  |  | DD difference between new and old film (New DD – Old DD) |  |  |  |  |  |  |  |
|  |  | B+F difference between new and old film (New – Old) |  |  |  |  |  |  |  |
|  |  |  |  |  | MD | DD | B+F |  |  |
|  |  | Old operating levels |  |  |  |  |  |  |  |
|  |  | Difference between new and old film |  |  |  |  |  |  |  |
|  |  | New operating levels |  |  |  |  |  |  |  |

### CROSSOVER WORKSHEET

#### EXAMPLE

| New Emulsion # |  | 24578 |  |  | Old Emulsion # |  | 23456 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Film # | Low Density (LD) Step # 10 | Mid Density (MD) Step # 11 | High Density (HD) Step # 13 | B+F | # Film # | Low Density (LD) Step # 10 | Mid Density (MD) Step # 11 | High Density (HD) Step # 13 | B+F |
| 1 | 0.49 | 1.25 | 2.39 | 0.18 | 1 | 0.46 | 1.27 | 2.33 | 0.17 |
| 2 | 0.50 | 1.23 | 2.43 | 0.18 | 2 | 0.48 | 1.30 | 2.30 | 0.17 |
| 3 | 0.49 | 1.26 | 2.40 | 0.17 | 3 | 0.46 | 1.27 | 2.28 | 0.18 |
| 4 | 0.53 | 1.28 | 2.41 | 0.18 | 4 | 0.48 | 1.28 | 2.32 | 0.17 |
| 5 | 0.49 | 1.28 | 2.43 | 0.18 | 5 | 0.47 | 1.31 | 2.35 | 0.18 |
| ###### Average | 0.50 | 1.26 | 2.41 | 0.18 | Average | 0.47 | 1.29 | 2.31 | 0.17 |
| Average Density Difference: DD = HD – LD = 1.91 |  |  |  |  | ###### Average Density Difference: DD = HD – LD = 1.84 |  |  |  |  |
|  |  | MD difference between new and old film (New MD – Old MD) |  |  |  |  | -0.03 |  |  |
|  |  | DD difference between new and old film (New DD – Old DD) |  |  |  |  | +0.07 |  |  |
|  |  | B+F difference between new and old film (New – Old) |  |  |  |  | +0.01 |  |  |
|  |  |  |  |  | MD | DD | B+F |  |  |
|  |  | Old operating levels |  |  | 1.34 | 1.90 | 0.17 |  |  |
|  |  | Difference between new and old film |  |  | -0.03 | +0.07 | +0.01 |  |  |
|  |  | New operating levels |  |  | 1.31 | 1.97 | 0.18 |  |  |

##### Figure 5A.  Crossover Worksheet